Leverage dial gauge

一、Its characteristics are as follows：

- The resolution is 0.001mm and easy to read；

- The rigid outer frame is extremely stiff and durable；

- The clear, concise, large-diameter dial keeps the reading error within a reasonable range；

- Anti-magnetic pointers and styli for reliable use in magnetic environments；

- The lid is coated with an anti-slip coating；

- Outer frame material: 4CR13 steel, good stability for long-term use, no deformation and no rust; The probe is made of precious stones, and the wear resistance reaches 50,000 times。

二、The functions are as follows：

- Flatness detection；（2）Parallelism detection；（3）Perpendicularity detection；（4）Verify the accuracy of other instruments。

三、Flat graphics and dimensional specifications

四、Measuring range and accuracy

Resolution and accuracy: The resolution is 0.001 mm, and the accuracy class is plus or minus 2 microns

| code | Measuring range | Graduationvalue | Dial reading | 10scale accuracy（+/-） | Repeatability | Lap accuracy | Probe category | Weight (g) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2204 | 0-0.5 | 0.001 | 0-50-0 | 0.002 | 0.002 | 0.005 | ruby | 45kg |
| 2205 | 0-1.5 | 0.001 | 0-50-0 | 0.002 | 0.002 | 0.005 | ruby | 45kg |
| 2206 | 0-0.8 | 0.001 | 0-50-0 | 0.003 | 0.003 | 0.005 | ruby | 45kg |
| 2207 | 0-1 | 0.001 | 0-50-0 | 0.003 | 0.003 | 0.005 | ruby | 45kg |